# Lab Assignment

School of Engineering and Technology
Course Code & Name: ENCS304 Computer Networks

Program Name: B.Tech CSE, AI ML, Data Science, Cyber, FSD, UX/UI

## Submission Guidelines

### General Requirements

- Submission Deadline: Assignments must be submitted within one week of the assignment's release date.

- Submission Platform: Submit via the Learning Management System (LMS).

- GitHub Link: Include a link to your GitHub repository in the LMS submission.

- Individual Submission: Each student must submit individually.

## Evaluation

- Total Marks: 5 marks

- Evaluation Metrics:

  - Originality: Independent work and unique presentation

  - Correctness: Accurate configuration and valid results

  - Completeness: All tasks and required outputs included

# Assignment 1: Basic Network Topologies Using Switches and Hubs

## Experiment Objectives:

In this experiment, students will create three small LAN designs in Cisco Packet Tracer to understand how network topology influences connectivity, fault tolerance, and performance. Students will configure end devices with IPv4 addressing and verify communication using ICMP.

## Learning Outcomes

- Create small networks in Packet Tracer using different topologies

- Assign IP addresses correctly and test connectivity

- Identify failure points in different physical layouts

- Compare topology behavior using simulation results

## Concepts Used

- Star / Hub-based / Loop (ring-like) layouts

- IPv4 addressing and subnetting

- ICMP ping

- Packet Tracer Simulation Mode

## Detailed Instructions

### Task 1: Star Topology (Switch)

- Place 1 switch + 4 PCs

- Connect all PCs to the switch

- Assign IPs (example): 192.168.10.1–4 /24

- Ping PC1 → PC2, PC3, PC4

### Task 2: Bus-like Topology (Hub)

- Replace switch with hub

- Keep same IP scheme

- Send pings simultaneously and observe packet flow

### Task 3: Ring-like Topology (Loop)

- Place 3 switches and connect them in a loop

- Attach 1–2 PCs to each switch

- Ping across switches and observe path

### Task 4: Failure Test

- Disconnect one link in ring-like design

- Test if communication still works and note observations

## Expected Output

- Successful ping results in each topology

- Screenshot of each topology

- Observation notes on failure behavior and performance

## Practical Applications

- LAN design for offices and labs

- Selecting topology based on cost and fault tolerance

- Understanding basic physical network planning

## Submission Format

- exp1_topologies.pkt (Packet Tracer file)

- output_exp1.txt (ping results + observations)

- report_exp1.pdf (objective, topology screenshots, conclusions)

- README.md (how to open and test)

## Rubrics for Evaluation

| Criteria | Max Score | Description |
| --- | --- | --- |
| Originality | 2 | Clean unique layout + good comparison notes |
| Correctness | 2 | Correct IP config + successful connectivity |
| Completeness | 1 | All tasks + files submitted |

Evaluator’s Signature: ______________________